CHAPTER 6

TESTING AND EVALUATION

6.1 Overview

This chapter presents the testing and evaluation of the VisionPay system. Testing was conducted at multiple levels to ensure that each component of the system worked correctly in isolation and that the integrated system functioned as intended. The evaluation covers the AI model's detection performance, API endpoint functionality, User Interface (UI) behavior, hardware accuracy, and overall system performance.

Testing was carried out systematically. Unit tests were performed on individual API endpoints using the FastAPI automatic documentation interface at /docs. Integration tests verified that the mobile app, backend, and database worked together correctly. Performance tests measured detection speed and API response time. Hardware tests validated load cell accuracy against known weights.

6.2 Testing Methodology

The testing methodology followed a bottom-up approach. Individual modules were tested before integration. The AI model was evaluated using the validation set metrics provided by the Ultralytics training framework. API endpoints were tested manually using the FastAPI Swagger UI and by sending HTTP requests with sample images. The Flutter application was tested on a physical Android 14 device.

Table 6.1: Testing Types and Scope

| Testing Type | Scope | Method |
| --- | --- | --- |
| Unit Testing | Individual API endpoints | FastAPI /docs + HTTP requests |
| Model Evaluation | YOLOv8n detection accuracy | Validation set metrics |
| Integration Testing | Mobile app + Backend + DB | End-to-end scan flow |
| UI Testing | All four app screens | Manual testing on Android device |
| Performance Testing | Detection speed, response time | Timed requests |
| Hardware Testing | Load cell accuracy | Known weights comparison |

6.3 API Endpoint Testing

All API endpoints were tested using the automatically generated Swagger documentation available at the /docs route of the deployed backend. Each endpoint was tested with valid inputs, edge cases, and invalid inputs to verify correct behavior. The results are summarized in the table below.

Table 6.2: API Endpoint Test Results

| Endpoint | Test Input | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- |
| POST /detect | Product image (JPG) | List of detected products | Products returned with prices | Pass |
| POST /detect | Empty image / no products | Empty products list | Empty list returned | Pass |
| POST /checkout | Product image | Bill + transaction saved | Bill generated correctly | Pass |
| GET /products | None | All 50 products | 50 products returned | Pass |
| GET /stats | None | Revenue and counts | Correct stats returned | Pass |
| GET /low-stock | None | Products below threshold | Low stock list returned | Pass |
| POST /product/add | New product data | Product added to DB | Product added successfully | Pass |
| DELETE /product/delete/{id} | Valid product ID | Product removed | Product deleted | Pass |
| GET /weight | None | Weight in grams and kg | Weight value returned | Pass |
| WS /ws | WebSocket connection | Real-time broadcast | Messages received on connect | Pass |

6.4 AI Model Evaluation

The YOLOv8n model was evaluated using the validation subset of the training dataset after 100 epochs of training. The primary evaluation metric used was mAP50, which measures the mean Average Precision at an Intersection over Union (IoU) threshold of 0.5. This metric reflects how accurately the model detects objects and draws bounding boxes around them.

6.4.1 Overall Model Performance

Table 6.3: Overall Model Performance Metrics

| Metric | Value |
| --- | --- |
| Overall mAP50 | 39.9% |
| Overall mAP50-95 | 27.3% |
| Precision | 61.2% |
| Recall | 48.7% |
| Detection Speed (per image) | < 1 second |
| Model Size | 6.2 MB |
| Training Epochs | 100 |

6.4.2 Per-Class Performance

Detection accuracy varied significantly across classes. Packed products with distinctive packaging achieved much higher accuracy compared to fruits and vegetables, which have high visual similarity and natural variation in appearance. The table below shows accuracy results for all classes.

Table 6.4: Per-Class mAP50 Results

| Class | Category | mAP50 (%) |
| --- | --- | --- |
| CocoMo | Packed Product | 99.5 |
| Lays French Cheese | Packed Product | 99.5 |
| Candi Biscuit | Packed Product | 95.3 |
| Chunkin Chocolate | Packed Product | 88.7 |
| Prince Biscuit | Packed Product | 82.1 |
| Tomato | Vegetable | 72.1 |
| Banana | Fruit | 68.4 |
| Potato | Vegetable | 61.7 |
| Onion | Vegetable | 58.3 |
| Apple | Fruit | 55.2 |
| Carrot | Vegetable | 52.8 |
| Capsicum | Vegetable | 49.6 |
| Cucumber | Vegetable | 47.3 |
| Watermelon | Fruit | 43.1 |
| Grapes | Fruit | 38.9 |
| Pineapple | Fruit | 35.4 |
| Ginger | Vegetable | 28.6 |
| Garlic | Vegetable | 24.3 |
| Mint | Vegetable | 19.7 |
| Average (all 53 classes) | — | 39.9 |

The lower accuracy on classes like Mint (19.7%) and Garlic (24.3%) is primarily due to the small size of these items in images and their visual similarity to other green or white vegetables. In practical use, the system still detects these items when they are clearly presented close to the camera, and the confidence threshold of 0.55 prevents misidentification in most cases.

6.5 System Integration Testing

Integration testing verified that all components of the system worked together as expected. The complete billing flow was tested end-to-end: a product was placed in front of the phone camera, the Detect button was pressed, the product appeared in the cart, the bill was generated, and the transaction was saved to the database. The same test was repeated with weight-based products to verify the weight dialog and load cell integration.

Table 6.5: Integration Test Scenarios

| Scenario | Steps | Expected | Result |
| --- | --- | --- | --- |
| Packed product detection | Place CocoMo in front of camera → Detect | Added to cart at Rs.20 | Pass |
| Vegetable detection | Place Tomato → Detect → Enter weight | Weight dialog opens, price calculated | Pass |
| Multiple products | Place 3 products → Detect | All 3 added to cart | Pass |
| Unknown object | Place random object → Detect | Nothing added to cart | Pass |
| WebSocket sync | Mobile scan → Check laptop browser | Cart updates on laptop | Pass |
| Load cell read | Place 500g weight → Read Scale button | ~500g displayed | Pass |
| Receipt generation | Complete cart → Checkout | Receipt shown, DB updated | Pass |
| Manager stats | After checkout → Manager dashboard | Revenue updated | Pass |

6.6 Performance Testing

Performance testing measured the time taken for the most critical operations in the system. All measurements were taken over 10 repeated trials and the average is reported.

Table 6.6: System Performance Results

| Operation | Average Time | Acceptable Threshold |
| --- | --- | --- |
| Image upload to /detect (mobile → HF Space) | 1.8 seconds | < 3 seconds |
| YOLO inference on server | 0.7 seconds | < 1 second |
| Database query (products) | < 50 ms | < 200 ms |
| Bill generation | < 30 ms | < 100 ms |
| WebSocket broadcast delay | < 100 ms | < 500 ms |
| Load cell reading update interval | 500 ms | < 1 second |
| App startup time (cold) | 2.1 seconds | < 5 seconds |

The total time from pressing the Detect button to seeing products in the cart was approximately 2.5 seconds on average, which is acceptable for a grocery billing counter environment. Network latency accounts for most of this time since the backend is hosted on a remote cloud server.

6.7 Hardware Testing

The load cell was tested using known calibration weights to verify measurement accuracy. Five different weights were placed on the scale and the readings were recorded.

Table 6.7: Load Cell Accuracy Test Results

| Actual Weight (g) | Displayed Weight (g) | Error (g) | Error (%) |
| --- | --- | --- | --- |
| 100 | 101 | ±1 | 1.0% |
| 250 | 252 | ±2 | 0.8% |
| 500 | 499 | ±1 | 0.2% |
| 750 | 753 | ±3 | 0.4% |
| 1000 | 998 | ±2 | 0.2% |

The load cell demonstrated an average error of less than 1% across all tested weights. This level of accuracy is more than sufficient for grocery billing purposes, where a margin of ±5 grams is typically acceptable.

6.8 UI Testing

The Flutter application was tested on a physical Android 14 device. All four screens were tested for layout correctness, navigation, and functionality. The camera preview was verified to cover the maximum area of the display using the FittedBox.cover configuration. The weight dialog was tested with both manual input and automatic scale reading. The manager dashboard was tested for data accuracy — revenue figures were verified against manually calculated totals from the transactions table.

6.9 Comparison with Existing Systems

VisionPay was compared against traditional barcode-based billing systems and a generic AI checkout system to highlight its advantages and limitations.

Table 6.8: Comparison with Existing Billing Systems

| Feature | Barcode System | Amazon Go | VisionPay |
| --- | --- | --- | --- |
| Hardware Cost | Low | Very High | Low |
| Barcode Required | Yes | No | No |
| Produce Weighing | Manual | Automatic | Semi-automatic |
| Mobile Camera Use | No | No | Yes |
| Internet Required | No | Yes | Yes |
| Works Offline | Yes | No | No |
| Deployment Complexity | Low | Very High | Low |
| Pakistani Product Support | Yes | No | Yes |
| Cost for Small Store | Affordable | Not Feasible | Affordable |